# Neuromorphic X2 wb Behavioral Model Guide

Customer interface and digital behavioral operation

Model identity  The delivered source file is Neuromorphic_X2_wb.sv and its single customer-facing module is Neuromorphic_X2_wb. The model exposes the X2 macro interface directly and does not require an extra wrapper.

| Item | Value |
| --- | --- |
| Source file | Neuromorphic_X2_wb.sv |
| Module | Neuromorphic_X2_wb |
| Wishbone register | 0x30000004 |
| Array abstraction | 32 rows by 32 columns |
| Regression status | 16 cases passed with zero failed checks |

Purpose and verification boundary

The model reproduces the digital behavior needed by customer simulations: configuration, Wishbone command handling, SET, RESET, READ, COMPUTE, response queuing, scan controls, and observable decoder signals. The analog and power pins remain available for interface compatibility.

This source is not a transistor-level or extracted-layout model. It does not replace analog verification, gate-level timing simulation, DRC, or LVS signoff for the physical macro.

External interface

| Group | Signals | Modeled behavior |
| --- | --- | --- |
| Power | VDDC1, VDDC2, VDDA1, VDDA2, VSS | Included when USE_POWER_PINS is defined |
| Clocks and resets | user_clk, user_rst, wb_clk_i, wb_rst_i | Wishbone state uses wb_clk_i and active-high wb_rst_i |
| Wishbone | wbs_* inputs, wbs_ack_o, wbs_dat_o | Queues commands and returns status or operation results |
| Scan and test | ScanInCC, ScanInDL, ScanInDR, TM, ScanOutCC, TM_o | Models scan shifting and scan-selected decoder outputs |
| Analog boundary | stop_bit_i and ten bias pins | Keeps the macro IO; does not model electrical rail behavior |
| Controller | pwm_pulse_o, mux_sel_o, ramp_o, v_count_o, precharge_o, hold_o, start_o | Shows abstract operation activity |
| Decoders | SL, BL, and WL data, address, and float buses | Shows one-hot functional or scan selections |

Internal behavioral constants

| Constant | Value | Meaning |
| --- | --- | --- |
| ADDR_MATCH | 0x30000004 | Wishbone command and result register |
| READ_DELAY | 160 clocks | Abstract READ latency |
| PROGRAM_DELAY | 360 clocks | Base SET or RESET latency |
| COMPUTE_DELAY | 450 clocks | Abstract COMPUTE latency |
| CONFIG_WRITES | 3 | Configuration packets required after reset |
| INITIAL_CELL_LEVEL | 0x0E23 | Initial digital cell value |

Wishbone command behavior

A transfer is selected when cyc and stb are high, all byte enables are set, and the address is 0x30000004. A write places one packet in the command FIFO. A read returns status, the oldest result, or 0xA0000000 when the response FIFO is empty.

| Bits | Field | Behavior |
| --- | --- | --- |
| 31 to 30 | Mode | 00 RESET, 01 READ, 10 COMPUTE, 11 SET |
| 29 to 25 | Row | Selects one row |
| 24 to 20 | Column | Selects one column |
| 19 | Error read | Returns the status code on a Wishbone read |
| 18 | Full column | Requests all 32 COMPUTE result columns |
| 17 | Last packet | Launches the accumulated COMPUTE operation |
| 16 to 8 | Reserved | Set to zero by the supplied testbench |
| 7 to 0 | PWM | Programming or compute value |

Result encoding

Every non-status result is encoded as:

{11'b0, mode[1:0], column[4:0], value[13:0]}

SET and RESET each update one modeled cell and automatically enqueue one readback. READ enqueues one result. COMPUTE returns the selected columns in ascending order; bit 18 expands the selection to all 32 columns.

Digital array and FIFOs

A 32 by 32 digital state array records whether each cell is set or reset. A 14-bit level is retained for every modeled cell and contributes to returned values.

The depth-32 command FIFO accepts Wishbone writes while a behavioral operation is delayed. The depth-32 response FIFO preserves result order, and direct pointer checks apply backpressure when a full-column result fills it.

Stop bit and observation outputs

stop_bit_i does not generate the Wishbone result. It only affects hold_o and v_count_o during an active READ or COMPUTE observation interval. The model treats an unconnected or unknown stop bit safely, so a RISC-V test can control the model entirely through Wishbone while stop_bit_i is tied low.

Power pin compilation

Compile normally for a digital-only simulation. Define USE_POWER_PINS when the testbench must expose VDDC1, VDDC2, VDDA1, VDDA2, and VSS. These pins preserve interface compatibility and do not implement an electrical supply model.