# Neuromorphic X2 wb Testbench Guide

Wishbone regression procedure and verified outputs

Testbench identity  The delivered file is Neuromorphic_X2_wb_tb.sv and its top module is Neuromorphic_X2_wb_tb. It instantiates Neuromorphic_X2_wb directly and runs all 16 verified cases by default.

| Item | Value |
| --- | --- |
| Testbench file | Neuromorphic_X2_wb_tb.sv |
| Top module | Neuromorphic_X2_wb_tb |
| DUT module | Neuromorphic_X2_wb |
| Clock | 10 MHz with a 100 ns period |
| Full regression | TOTAL=16 PASS=16 FAIL=0 FAILED_CHECKS=0 |

Testbench operation

The testbench drives every functional command through the Wishbone interface. Its read and write tasks wait for wbs_ack_o, so bus completion is based on the DUT handshake rather than a fixed transaction delay. The analog bias pins are tied low because this regression checks digital behavior.

selected_case defaults to 0 and runs the complete 16-case suite.

The +CASE=<1..16> plus argument runs one selected case.

SET and RESET checks wait 400 clocks before reading the automatic readback.

The stop-bit process follows the active BL address after ramp assertion.

Private DUT state is used only for compute-mask, result-count, and FIFO-full checks.

Compile and run

Compile both SystemVerilog files and select the testbench top module:

iverilog -g2012 -s Neuromorphic_X2_wb_tb -o simv \
  Neuromorphic_X2_wb.sv Neuromorphic_X2_wb_tb.sv
vvp simv +CASE=0

Run one case by changing the plus argument, for example:

vvp simv +CASE=14

Wishbone sequence

Apply active-high reset and return the Wishbone master signals to idle.

Write the three configuration packets to address 0x30000004 with all byte enables asserted.

Write operation packets using the required mode, row, column, control bits, and PWM value.

Read the same address after the required operation latency and check each returned word.

Check the 0xA0000000 empty marker after all expected FIFO entries have been consumed.

Verification cases and results

| Case | Purpose | Significant output | Result |
| --- | --- | --- | --- |
| 1 | Three configuration packets | Configuration accepted; no result read | PASS |
| 2 | Configure and read empty FIFO | 0xA0000000 | PASS |
| 3 | Reset and read empty FIFO | 0xA0000000 | PASS |
| 4 | SET and RESET automatic readback | 0x0019323D, 0x00012324 | PASS |
| 5 | Six COMPUTE packets without launch | 0xA0000000 | PASS |
| 6 | Full-column COMPUTE | 32 words from 0x00100000 through 0x0017C000 | PASS |
| 7 | Program readbacks followed by empty reads | SET and RESET results, then five empty reads | PASS |
| 8 | Three reads after each program phase | One result and two empty reads per phase | PASS |
| 9 | Independent SET and RESET readbacks | 0x0019323D and 0x0001A324 | PASS |
| 10 | Six rows and one compute column | 0x0011C000, then empty | PASS |
| 11 | Four selected compute columns | Four ordered results, then empty | PASS |
| 12 | Three-packet full-column COMPUTE | 32 ordered column results, then empty | PASS |
| 13 | Six program operations and five computes | 11 ordered results, then empty | PASS |
| 14 | Program results plus full-column COMPUTE | Four program words and 32 compute words | PASS |
| 15 | Two full-column COMPUTEs with backpressure | Two complete 32-word result sets | PASS |
| 16 | Imported SET RESET and READ sequence | 0x00187339, 0x0001641A, 0x0008241A | PASS |

Key mixed operation outputs

Case 13 returned these eleven words before the empty marker:

0018F28B  001AF28C  001D328D  0000E37C  0002E37D  0006637E
00108000  00114000  00124000  00138000  0015C000

Case 14 returned 001932BB, 001B32BC, 000123AC, and 0004A3AD before the 32 ordered column words. Case 15 returned two full-column result sets; the second included 001080D1 at column 2 and 0013C0D2 at column 15.

Pass criteria and final result

Each case reports TEST PASS only when its Wishbone results, FIFO behavior, masks, and operation counts match the expected values. The verified run completed with the following summary:

FINAL SUMMARY: TOTAL=16 PASS=16 FAIL=0 FAILED_CHECKS=0

The testbench validates the behavioral model's digital interface. It does not validate analog circuit behavior, gate-level timing, DRC, or LVS.